The goal of the assignment is to develop an understanding for the different Verilog language concepts and the respective constructs associated with this HDL.

Homework 2: PART 1

The following code was typed into Quartus II, and then synthesized in its own project (HW3Pt1):

// Carlos Lazo

// ECE574

// Homework #2

// This file is the blocking file provided for Part 1.

module HW2Pt1 (input clk, input [7:0] iNB, iB,

output reg [7:0] aNB, bNB, aB, bB);

always @(posedge clk) begin

aNB <= iNB;

bNB <= aNB;

end

always @(posedge clk) begin

aB = iB;

bB = aB;

end

endmodule

As requested, the next page showcases the RTL View of the synthesized circuit.

This design makes sense for the following reasons –

Blocking (aB & bB):

Setting these as blocking assignments (aB = iB and bB = aB), aB was first set to iB.  Following this logic, bB is always being set to aB (with no delay), but after aB is set to iB.  The RTL View shows aB and bB both being set to iB[7:0], which is correct from a blocking, logic functionality perspective.

Non-Blocking (aNB & bNB):

Setting these as blocking assignments (aNB <= iNB and bNB <= aNB), the logical assignments happen concurrently, with absolutely no delay.  Thus, when the clk hits a positive edge, memory is being assigned from iNB into aNB at the same time that aNB is being assigned into bNB.  Thus, the RTL View shows the Q, from aNB, feeding into D of bNB, which is correct following the logic stated above.

RTL View of Synthesized B/NB Circuit

The following is the Verilog Test Bench I developed in order to analyze the timing differences between the Pre and Post Synthesis versions of the B/NB Verilog code:

// Carlos Lazo

// ECE574

// Homework #2, Part 1

// This file is the tester for the Pre & Post synthesis

// simulations of the Blocking & Non-Blocking assignments.

`timescale 1ns/100ps

module HW2Pt1Tester;

reg clk;

reg [7:0] input_B_NB;

// Define all outputs as wires.

wire [7:0] aNB_Pre , bNB_Pre , aB_Pre ,  bB_Pre;

wire [7:0] aNB_Post, bNB_Post, aB_Post, bB_Post;

// Instantiate both Pre_Synthesis and Post_Synthesis

// versions of the Blocking & NonBlocking code.

HW2Pt1   Pre_Synth  (clk, input_B_NB, input_B_NB,

aNB_Pre , bNB_Pre , aB_Pre ,  bB_Pre);

HW2Pt1PS Post_Synth (clk, input_B_NB, input_B_NB,

aNB_Post, bNB_Post, aB_Post, bB_Post);

// Set clock to start at 0 and input_B_NB to all 0's.

initial begin

clk = 0; input_B_NB = 0;

end

// Repeat the clock for a total of 50 cycles.

initial repeat (50) #7 clk = ~clk;

// Create a pseudo-randomized input changing sequence that flips

// the input bits at different frequencies.

initial repeat (20) #15  input_B_NB   [2] = ~input_B_NB [2];

initial repeat (18) #20  input_B_NB [8:6] = ~input_B_NB [8:6];

initial repeat (30) #10  input_B_NB [4:3] = ~input_B_NB [4:3];

endmodule

The following image, on the next page, showcases a zoomed in portion of the waveform output of my simulated Test Bench.  I will highlight specific sections, which are attributed to the specificity involved with the Blocking and Non-Blocking assignments.

Pseudo-Randomized Waveform Input Sample

RED Box:  This box showcases the timing delay between the Pre and Post-Synthesis instantiations of the Blocking/Non-Blocking code.  At that positive edge of the clock, the output (00011000) is immediately read into aNB_Pre, and is delayed by about 6 ns (almost one full clock cycle).

GREEN Box: This box demonstrates the concurrency between aB_Pre & bB_Pre.  Based on the way the circuit was synthesized, both of the inputs will be set to input_N_NB immediately, logically speaking.

BLUE Box: This set of boxes highlights the issue that occurs with the Non-Blocking assignments.  aNB_Pre reads in the input correctly from input_b_NB (11011000), but as this is happening, bNB_Pre is reading in the current value of aNB_Pre, which happens to be the previous input.  With this instantaneous reading of inputs with no delay, bNB_Pre ends up not being set to the correct input, as expected.  These sorts of errors can occur with Non-Blocking statements, especially if they happen concurrently without any sort of timing delay.

The following pages go into the Technology View of the synthesized circuit, which highlight the primary data components of the module:

Technology View of aB Data Element

Technology View of aNB Data Element

Technology View of bB Data Element

Technology View of bNB Data Element

The Technology View of all the main components line up with the RTL seen earlier.

The following is a view of the board layout as populated by Quartus II:

Chip Planner View of Blocking/Non-Blocking Module

The Test Bench, along with all the scenarios showcased and the different views, create an avid and overall encompassing representation of the differences between Blocking an Non-Blocking procedural assignments in the Verilog HDL.

Homework 2: PART 2

Seen below is the code I developed to model the memory in Part 2 of the homework, along with the Test Bench I developed to tests the different aspects.

// Carlos Lazo

// ECE574

// Homework #2

// This is the module for the memory modeling exercise

// for Homework #2 Part 2.

`timescale 1ns/100ps

module MemoryModel (input clk, read, write,

input [7:0] inbus,

input [9:0] addrbus,

output reg [7:0] outbus);

// Define the memory, words = 8 bits and an address space = 1024.

reg [7:0] myMem [0:1023];

// The read input must be asserted asynchronously, so always

// check to see when it changes state.

always @(read) begin

// If read was just set to 1, then perform the read operation.

if (read)

outbus <= myMem[addrbus][7:0];

end

// The write input must be asserted synchronously with the clock,

// so first check to see if we're at the rising edge.

always @(posedge clk) begin

// If write is currently asserted as 1, then perform write

// into the memory block.

if (write)

myMem[addrbus][7:0] <= inbus;

end

endmodule

// Carlos Lazo

// ECE574

// Homework #2

// This is the module for the memory modeling Test Bench

// for Homework #2 Part 2.

`timescale 1ns/100ps

module MemoryModelTester;

// Define all variables needed for the tester.

reg clock, read, write;

reg [7:0] inMem;

reg [9:0] addr;

wire [7:0] outMem;

// Instantiate a variable of type MemoryModel.

MemoryModel UUT (clock, read, write,

inMem, addr, outMem);

// Initialize all inputs respectively.

initial begin

clock = 0;

read  = 0;

write = 0;

inMem = 0;

addr  = 0;

end

// Change the clock input every 10 ns.

// This timing will make it easy to develop test scenarios.

initial repeat (10) #10 clock = ~clock;

// Create the testing scenario:

initial begin

// Scenario 1 - Test simple writing and reading capability.

//              At 5ns, set write = 1 and define an address.

//              At 10ns, clock will be at rising edge, and

//              write will be asserted. At 15ns, assert read

//              and outbus should display contents at that

//              address. Reset read & write back to 0.

inMem <= #5  8'b00001111;

write <= #5  1;

addr  <= #5  10'd1000;

read  <= #15 1;

read  <= #16 0;

write <= #16 0;

// Scenario 2 - Write a different 8-bit value during the next 3

//              rising edges of the clock, but to different registers

//              within the memory block. Intermix read statements at

//              varying VALID addresses.

write <= #20 1;

// Write happens at #30.

inMem <= #20 8'b10101010;

addr  <= #25 10'd500;

// Assert read at address 500 at #35.

addr  <= #30 10'd500;

read  <= #35 1;

read  <= #36 0;

// Write happens at #50.

inMem <= #40 8'b11011011;

addr  <= #45 10'd250;

// Assert read at address 250 at #55.

addr  <= #50 10'd250;

read  <= #55 1;

read  <= #56 0;

// Write happens at #70.

inMem <= #60 8'b00111010;

addr  <= #65 8'd100;

// Assert read at address 500 at #75.

// Ensures that contents are still there.

addr  <= #60 10'd1000;

read  <= #75 1;

read  <= #76 0;

write <= #80 0;

// Scenario #3 - This is an attempt to try and write in only 4 bits of

//               data into the memory block instead of the regular 8 bits.

// Write happens at #90.

write <= #85 1;

addr  <= #85 10'd777;

inMem <= #85  4'b1111;

// Assert read at address 777 at #91.

read  <= #91 1;

read  <= #92 0;

end

endmodule

Attached below is a screen capture of the Test Bench Waveform, with explanations pertaining to the comments expressed in the code above:

Memory Model Test Bench Waveform Output

In looking at what the module could and could not handle, in regards to different bit and slice selections:

- Crash occurred when addrbus  decimal values were out of the range of 0 – 1023

  - This could be corrected with error detection in the Verilog code

- If assigning addrbus values less than 8 bits, Verilog adds leading 0’s to the front of the number and forces it to match the size of the declared memory space.

Through guided timing execution of certain commands in my Test Bench (as seen through the comments), and through correlation with this waveform, the read and write commands are executed correctly in this instantiated memory block, local to the MemoryModel module.

Homework 2: PART 3

See code located in Pt3 of the .zip file, TextIO.v, for task instantiation and corresponding comments.

No Test Bench was made as it wasn’t called for, but code does compile and constructs are built correctly with assumptions well documented.